Installing Raspbian OS on a MicroSD Card

This document outlines the process of installing the Raspbian OS on a microSD card for use in Raspberry Pi applications

Bill of Materials:

- Personal Computer with standard web browser (1)

  - A USB-A port is helpful (if one is not available, a USB-A to a port of your choice adaptor will be needed)

- MicroSD card (1)

- SD card adaptor (1)

  - Must be able to connect microSD to SD

- SD card reader to USB-A (1)

- Raspberry Pi (1)

On your personal computer:

- Navigate to Raspberry Pi’s Software page (https://www.raspberrypi.com/software/)

Figure 1: Software landing page (photo taken in 2025)

- Download the imager that matches the personal computer’s operating system (Windows, Mac, etc)

- Open the imager

  - The personal computer’s OS may create warning. If this occurs, read it and allow the imager to open

  - See Figure X for running imager

Figure 2: The Raspberry Pi Imager (macOS)

- Select the appropriate device

Figure 3: Device selection window

- Select desired operating system

Figure 4: OS Selection Menu

- Select microSD card for writing

  - WARNING: You will overwrite the entire microSD card in the imaging process

Figure 5: Storage menu

- Begin imaging

  - This could take about an hour

Once imaging is completed and verified:

- Eject the SD card writer

- Remove the microSD card

- Insert microSD card into the Raspberry Pi

- Plug in peripherals (keyboard, mouse, monitor, etc)

- Power the Raspberry Pi

Once the Raspberry Pi is powered on:

- Fill out location settings

- Create your user account

- Connect to WiFi

- Choose a preferred web browser

- The system will prompt an update check. Allow this procedure

- Allow the Raspberry Pi to restart

- Upon restarting, the Raspberry Pi should have the desired OS and be ready for use